CEM Discover Microwave Reactor — Dry Run (Clickthrough Only)

Goal: load empty sealed vessel, run minimal heating program, save method  ·  No reagents

SUPPLIES (minimal)

| ☐ | Empty 10 mL CEM pressure vial + cap + septum |
| --- | --- |
| ☐ | Stir bar (small) |
| ☐ | USB stick |

Do NOT run the microwave with an empty vial containing NO contents and NO solvent at high power — you can damage the cavity. Use minimal power (10 W) for the dry run.

PRE-VISIT CHECK

| ☐ | Compressed air supply on (cooling) |
| --- | --- |
| ☐ | Cooling fan + IR temperature sensor functional |
| ☐ | Front loading door closes and latches |

POWER UP

| ☐ | Power on the Discover unit (rear rocker) |
| --- | --- |
| ☐ | Wait for self-test to complete (~1 min) |
| ☐ | Launch Synergy / ChemDriver software on PC |
| ☐ | Verify instrument shows "Connected / Ready" |

SOFTWARE CLICKTHROUGH

| ☐ | New Method → Standard mode |
| --- | --- |
| ☐ | Target temperature: 50°C · Hold time: 30 sec · Power Max: 10 W |
| ☐ | Stirring: medium |
| ☐ | Cooling: ON during run |
| ☐ | Place empty capped vial (with stir bar) into the cavity, close door |
| ☐ | Click Start — power ramps, vial warms slightly, holds 30 sec, cools |
| ☐ | Watch temperature + pressure traces in real time |
| ☐ | When complete, Save Method, Save Run Data → .csv to USB |

SHUTDOWN

| ☐ | Wait for vial to cool below 30°C |
| --- | --- |
| ☐ | Open door, remove vial |
| ☐ | Close software; power off Discover |
| ☐ | Pack out |

TIMING & BOTTLENECK LOG

| Step | Time | Notes / bottleneck |
| --- | --- | --- |
| Power up + self-test |  |  |
| Software online |  |  |
| Method setup |  |  |
| Run (heat 50°C, 10 W, 30 sec) |  |  |
| Cool + unload |  |  |
| Save method + data |  |  |
| TOTAL on-site time |  |  |

Bottlenecks / supplies that ran short / things to add to next Amazon order: